Greenhouse Gas Emissions Methodology Report

2024

Contents

Overview	2

Our emissions	3

Organisational boundary	3

Scope 1 and Scope 2: methodology overview	4

Scope 1	4

Scope 2	5

CAML’s Scope 3: methodology and summary	6

Scope 3	6

Definition	6

Approach	6

Scope 3 summary emissions	1

Scope 3 emissions methodology details	2

Category 1 - Purchased goods and services	2

Category 2 – Capital goods	5

Category 3 – Fuel and energy related activities	7

Category 4 – Upstream transportation and distribution	9

Category 5 – Waste generated in operations	11

Category 6 – Business travel	13

Category 7 – Employee commuting	15

Category 8 – Upstream leased assets	17

Category 9 – Downstream transportation & distribution	18

Category 10 – Processing of sold products	20

Category 11 – Use of sold products	21

Category 12 – End-of-Life treatment of sold products	23

Category 13 – Downstream leased assets	25

Category 14 – Franchises	26

Category 15 – Investments	27

Appendix 1 - Glossary of terms	28

Appendix 2 - Units of measurement	30

Appendix 3 – The Scope 1 and 2 emission sources	31

Table 3.1 – The Sources of CAML’s Scope 1 emissions	31

Table 3.2 – Sources of Scope 2 emissions	31

Appendix 4 – References	32

# Overview

This Methodology Report outlines the approach, assumptions, calculation boundaries and reference sources for disclosing the greenhouse gas (GHG) emissions of the Central Asia Metals PLC (‘CAML’) group for the year 2024, alongside a comparative analysis.

This report, in combination with the published data on Scope 1, Scope 2 and Scope 3 emissions in relation to our business, meets the disclosure requirements of the Global Reporting Initiative (GRI) Universal standard GRI 305: Emissions, and is aligned with the recommendations of the Financial Stability Board’s Task Force on Climate-related Financial Disclosures (TCFD).

Estimations of GHG emissions follow methodologies in line with the GHG Protocol: A Corporate Accounting and Reporting Standard, enriched with further insights from the GHG Protocol: Scope 2 Guidance, the Corporate Value Chain (Scope 3) Accounting and Reporting Standard, and the Technical Guidance for Calculating Scope 3 Emissions. The GHG Protocol facilitates voluntary GHG emission calculation and reporting for companies, encompassing six key greenhouse gases as identified in the Kyoto Protocol:

- carbon dioxide (CO2)

- methane (CH4)

- nitrous oxide (N2O)

- hydrofluorocarbons (HFCs)

- perfluorocarbons (PFCs)

- sulphur hexafluoride (SF6)

For CAML, the relevance of PFCs and SF6 in the emissions inventory is typically considered non-essential, as these are not predominantly generated in its operations.

For gases other than CO2, a specific Global Warming Potential (GWP) is applied to account for their higher warming effect, resulting in a measure known as carbon dioxide equivalent (CO2e). The GWP is a metric that compares the energy the emissions of 1 tonne of a gas will absorb over a given period, relative to the emissions of 1 tonne of CO2, thereby providing a unified scale for understanding and reporting the impact of different greenhouse gases.

# Our emissions

# Organisational boundary

CAML's methodology for estimating GHG emissions encompasses all facilities under its operational control, including emissions from contractors at these sites. However, emissions from CAML’s corporate and regional offices, as well as exploration sites, are excluded from the Scope 1 and 2 GHG emissions inventory based on materiality considerations. When assessing Scope 3 GHG emissions, each category is individually evaluated for relevance and inclusion for the Scope 3 GHG emissions inventory. This assessment leads to the omission of leased assets, franchises, and investments, ensuring a focused and relevant emissions reporting.

# Scope 1 and Scope 2: methodology overview

## Scope 1

#### Definition

Scope 1 emissions encompass direct GHG emissions originating from CAML’s own activities or sources under its operational control.

#### Calculation boundary

The calculation boundary has been defined on an operational control basis and from sources we own and control.

- Mobile fleet combustion – Emissions arising from combustion as part of working machines and internal transport. These are mine haul trucks, drilling machines, transport vehicles etc. and emit carbon dioxide (CO2), methane (CH4) and nitrous oxide (N2O) emissions from vehicles.

- Combustion or detonation of explosive materials - The explosives used at Sasa mine are Ammonite explosives containing ammonium nitrate (NH4NO3), trinitrotoluene (TNT) and diesel fuel, releasing CO2 and NOx.

- Stationary combustion – Emissions resulting from combustion of fossil fuels, such as coal at our boiler houses in Kounrad.

- Fugitive Emissions - Hydrofluorocarbon (HFC) emissions from refrigeration and air conditioning equipment.

#### Calculation methodology

For Scope 1 emissions the data is collected from various sources:

##### Fleet mobile combustion

The combustion from our fleet is calculated by collecting data on the total litres of gasoline and diesel consumed for all our fleet across the operations for both surface and underground (UG) vehicles, as well as any fuel cards for employees. This figure is then converted into gigajoules (GJ) to calculate the total fossil fuel emissions in metric tonnes of carbon dioxide equivalent (tCO2e).

##### Combustion or detonation of explosive materials

The explosives used at Sasa mine are Ammonite explosives which contain ammonium nitrate, TNT and diesel fuel and the detonators use a base charge of Pentaerythritol tetra nitrate (PETN) as well as a primary charge. The total consumed explosives is converted from tonnes to GJ before calculating the associated emissions in tCO2e.

##### Combustion of fossil fuels for energy

The total tonnes of coal used at the boiler house in Kounrad is recorded based on volume of loading into boilers. The recorded value is converted to GJ before being used in emission tCO2e calculations. In addition, the Sasa site has the diesel generator for the emergency electricity supply, which can contribute to the fuel usage.

##### Fugitive emissions

The leakage rate of the type of refrigerator, based on the refrigerant units we have, incorporates a conversion factor to calculate the CO2 emissions (tCO2e).

##### Others

Water treatment plant, immaterial amounts of propane in the laboratory, limestone use around the dumps.

Table 1 – CAML’s Scope 1 Emissions

|  | Kounrad |  |  | Sasa |  |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 |
|  | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e |
| Coal | 19,407 | 18,096 | 17,935 | - | - | - | 19,407 | 18,096 | 17,935 |
| UG fleet | - | - | - | 3,469 | 3,010 | 2,837 | 3,469 | 3,010 | 2,837 |
| Surface fleet | 844 | 1,030 | 966 | 124 | 116 | 97 | 968 | 1,146 | 1,063 |
| Diesel generator | - | - | - | 3 | - | - | 3 | - | - |
| Explosives | - | - | - | 131 | 128 | 126 | 131 | 128 | 126 |
| Sewage treatment plant | - | - | - | 77 | 77 | 77 | 77 | 77 | 77 |
| Refrigeration | - | - | - | 19 | 19 | 13 | 19 | 19 | 13 |
| Total | 20,251 | 19,126 | 18,901 | 3,823 | 3,350 | 3,150 | 24,074 | 22,476 | 22,051 |

## Scope 2

#### Definition

Scope 2 emissions involve indirect GHG emissions stemming from purchased energy sources, such as electricity.

#### Calculation boundary

CAML’s emissions include purchased electricity from the grid for its operations.

#### Calculation methodology

The total electricity consumption (MWh) across the Group is recorded using meter readings across the sites and is checked against the energy suppliers’ invoices. The consumption value is converted into GJ to calculate the total CO2 emissions. Our Scope 2 emissions are calculated using the location-based method. At Sasa, all electricity comes from renewable sources, so emissions from electricity use are considered as zero. At Kounrad, part of the electricity is supplied by an on-site solar panel farm, which also does not contribute to Scope 2 emissions.

Table 2 – Scope 2 emissions

|  | Kounrad |  |  | Sasa |  |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 |
|  | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e | tCO2-e |
| Electricity | 30,985 | 35,669 | 37,031 | - | - | - | 30,985 | 35,669 | 37,031 |
| Total | 30,985 | 35,669 | 37,031 | - | - | - | 30985 | 35,669 | 37,031 |

# CAML’s Scope 3: methodology and summary

## Scope 3

## Definition

Indirect GHG emissions that occur in the value chain of a company but are not directly owned or controlled by that company. These encompass all indirect emissions associated with its operations in its upstream and downstream activities.

In 2024, CAML completed its third assessment of Scope 3 emissions across the group, covering data from January 1, 2024, to December 31, 2024. This Methodology Report outlines the Scope 3 assessment approach for 2024, including key accounting assumptions, supporting literature, and defined organisational and accounting boundaries. It also details the data sources and references used to estimate emissions from CAML's activities. The Scope 3 emissions assessment results presented in this report are part of our ongoing commitment to align with the TCFD recommendations, demonstrating transparency in communicating our strategy for evaluating and managing climate-related risks and opportunities.

Our 2024 methodology builds on the Greenhouse Gas Protocol’s Scope 3 guidance. We have introduced a more accurate emissions database and applied refined, category-specific assumptions to enhance data quality and calculation precision. While the core methodology remains unchanged, updated emission factors have significantly improved reporting accuracy.

CAML captures emissions across eleven of the fifteen categories, excluding those assessed as not material to the business. Notably, to enhance transparency, we also include categories 11 and 12  covering additional downstream activities outside of our control , which are not typically reported by our peers. Given the inherent variation in data and approaches, comparisons between companies remains limited. Our focus remains on producing decision-useful insights tailored to our operations and impact areas.

## Approach

The Methodology Report follows the guidelines set by World Business Council Sustainable Development (WBCSD) and World Resources Institute (WRI) Greenhouse Gas Protocol Corporate Value Chain (Scope 3) Accounting and Reporting Standard (the GHG Protocol) as amended. Additionally, it adheres to the Required Greenhouse Gases in Inventories: Accounting and Reporting Standard Amendment.

CAML’s Scope 3 assessment, disclosed within this report, meets the reporting requirements established by the GHG Protocol Technical Guidance for Calculating Scope 3 Emissions (Scope 3 Guidance) and the GHG Protocol1. According to the GHG Protocol, the calculation and reporting of an organisation’s Scope 3 should follow the principles of relevance, completeness, consistency, transparency, and accuracy. This report highlights any distinct trade-offs made and provides rationale behind them.

For Scope 3, the data presented reflects the activities related to CAML’s two primary subsidiaries and their operational sites: Kounrad and Sasa. Other entities, including minority-owned subsidiaries, have been excluded based on criteria of operational control or materiality. These exclusions are in line with the GHG Protocol and reflect the current availability and relevance of emissions data for these entities.

Inflation and Exchange Rate Adjustment

For all spend-based estimations in the Scope 3 inventory, the emission factors are based on 2019 US dollars. To align with the GHG Protocol Scope 3 Calculation Guidance, these spend values are adjusted to account for market value differences between 2019 and the reporting period. This is done using inflation rates and exchange rates, ensuring the emission factors are applied accurately. The adjustment process involves deflating the spend values for the current year. This deflation uses a ratio of the Chain-Type Price Indexes for Gross Output by Industry. from 2019 compared to the current year. These Price Index values are sourced from the US Bureau of Economic Analysis, with the assumption that all expenditures are subject to inflation in relation to the USD. Exchange rates used in the calculations are obtained from the internal financial department.

## Scope 3 summary emissions

CAML’s Scope 3 emissions for 2024 and the prior year comparison are summarised as follows:

Table 3 - Scope 3 emissions summary

| Category |  | GHG emissions |  |  |
| --- | --- | --- | --- | --- |
|  | Description | 2024 | 2023 | 2022 |
|  |  | tCO2e | tCO2e | tCO2e |
| Upstream | 1. Purchased goods & services | 31,538 | 27,115 | 23,443 |
|  | 2. Capital goods | 3,496 | 11,415 | 7,863 |
|  | 3. Fuel & energy related activities | 5,976 | 6,382 | 6,562 |
|  | 4. Upstream transport and distribution | 1,941 | 1,993 | 1,805 |
|  | 5. Waste generated in operations | 480 | 390 | 395 |
|  | 6. Business travel | 293 | 281 | 233 |
|  | 7. Employee commuting | 265 | 240 | 254 |
|  | 8. Upstream leased assets* | - | - | - |
| Downstream | 9. Downstream transport & distribution | 14,573 | 16,025 | 11,948 |
|  | 10. Processing of sold products | 98,659 | 115,647 | 120,010 |
|  | 11. Use of sold products | 58,339 | 62,708 | 63,908 |
|  | 12. End-of-life treatment of sold products | 25,211 | 27,632 | 28,952 |
|  | 13. Downstream leased assets* | - | - | - |  |
|  | 14. Franchises* | - | - | - |  |
|  | 15. Investments* | - | - | - |  |
| Total Scope 3 GHG emissions |  | 240,771 | 269,828 | 265,373 |  |

* Categories 8, 13, 14 & 15 show zero emissions as they are either not applicable or material to our operations.

## Scope 3 emissions methodology details

### Category 1 - Purchased goods and services

| Category 1 |  |
| --- | --- |
| Description | Upstream GHG emissions from the extraction, production, and transportation of goods and services purchased or acquired by CAML during the year. |
| Emissions (tCO2e) | 31,538 (2023: 27,115) |
| Calculation method | Spend-based approach |
| Activity data source | CAML accounting software data of all Opex supplier payments (pre-tax) categorised according to industry categories. |
| Emissions data source | Exiobase database and the World Input – Output Database GHG evaluator. |

#### Boundaries

This category covers emissions upstream of CAML’s operations associated with the extraction, production, and transportation of goods and services procured in the reporting year. Emissions related to fuel and energy activities, upstream transportation, business travel, and employee commuting are excluded, as these fall into separate categories (categories 3, 4, 6, and 7, respectively), as per Scope 3 Standard recommendations.

#### Calculation method

CAML total operating expenditure (opex) in US dollars for the year ended 31 December 2024 was extracted from our accounting software and categorised into industry according to Exiobase database. This opex spend was then discounted back to a 2019 equivalent spend, and this USD figure multiplied by an emissions factor provided by the Exiobase and the World Input – Output Database GHG evaluator, which is an online tool that provides a tonne of CO2-eq/$ spend that can be applied to supplier payments.

The adjusted spending for 2019 was multiplied by the respective industry emission factor, initially yielding the total emissions in kgCO2e and then converted to tCO2e in the summary of emissions.

#### Assumptions

Opex Data from at least 98% of our operations was categorised into distinct purchase types based on knowledge and assumptions of invoices. These categories were standard goods, services and a category termed 'mixed'. The 'mixed' label was used for suppliers delivering a diverse array of products or services, such as drilling companies offering both drilling fluids and contractor services. These purchase types were further classified under respective industries, aligning with the classifications by Exiobase.

Following the categorisation process, opex data was populated labelling each expenditure into a specific category. To maintain data integrity, all industry-related expenditures were combined and then adjusted through a pricing index2, reconciling 2024 values with those from 2019 to match the emission factor's baseline year. It has been assumed that spend-based emissions factors from the Exiobase are representative of the organisations, geographies and activities associated.

Assumptions were made to relate the procurement system spend classifications, based on an interpretation of the typical activities covered by that classification, to an appropriate emissions factor derived from Exiobase.

The comparative data from previous Scope 3 assessments (2022 and 2023) follows the same methodology applied in 2024 for calculating emissions based on OPEX spend. However, the emission factors used for 2022 and 2023 were sourced from Quantis Scope 3 Evaluator tool, which was in use at the time but has since been discontinued. The emission factors used to calculate the 2022 and 2023 emissions from purchased goods and services were not updated, as any difference was deemed minimal and not expected to have a material impact.

#### Inclusion and exclusion

The opex spend data from CAML’s accounting software is accurate, with analysis encompassing 98-100% of expenditures per operation.

This category reflects purchased goods and services data associated with CAML’s operational sites: Kounrad and Sasa.

The following entities are excluded from the Scope 3 Category 1 assessment, based on considerations of operational relevance, data availability, and materiality:

- CAML PLC (Head Office): Expenditure was excluded due to its limited contribution to total procurement-related emissions and overall immateriality in this category.

- CAML Exploration (‘CAML X’): An 80%-owned subsidiary established in early 2024 to conduct early-stage base metal exploration in Kazakhstan. CAML X’s activities are currently limited in scale and therefore considered immaterial for the purposes of this assessment.

- Aberdeen Minerals (‘Aberdeen’): An associate in which CAML holds a 28.4% interest. As CAML does not have operational control over Aberdeen and its emissions contribution is minimal, it has been excluded in line with the control-based boundary approach.

The Quantis Scope 3 Evaluator tool is no longer supported and the WRI Scope 3 evaluator tool has been discontinued, prompting a transition to Exiobase for 2024 year’s emissions disclosure. Exiobase was chosen for its broad geographic coverage and detailed industry data, making it well-suited for spend-based emission factor estimates.

### Category 2 – Capital goods

| Category 2 |  |
| --- | --- |
| Description | GHG emissions from capital goods purchased by CAML in the reporting year. Capital goods are final products that have an extended life. |
| Emissions (tCO2e) | 3,496 (2023: 11,415) |
| Calculation method | Spend-based approach |
| Activity data source | CAML accounting software data of all Capex supplier payments (pre-tax), according to industry categories. |
| Emissions data source | Exiobase database and the World Input – Output Database GHG evaluator. |

#### Boundaries

This category covers emissions upstream of CAML’s operations associated with capital goods procured in the reporting year. Similar to category 1 emissions, the emissions related to fuel and energy activities, upstream transportation, business travel, and employee commuting are accounted in separate categories, as per Scope 3 Standard recommendations.

#### Calculation method

The total Capital expenditure (capex) spend in US dollars for the year ended 31 December 2024 was extracted from our accounting software, categorised into industry according to Exiobase. This capex spend was then discounted to its 2019 equivalent spend, which was then multiplied by the relevant industry emission factor from the Exiobase and the World Input using the Output Database GHG evaluator (an online tool that provides a tonne of CO2-eq/$ spend that can be applied to supplier payments), yielding total emissions in kg CO2e and then converted to tCO2e in the summary of emissions.

#### Assumptions

Capex data from at least 98% of our operations was delineated into distinct purchase types: capital goods.

Following the categorisation process, capex data was populated labelling each expenditure into a specific category. To maintain data integrity, all industry-related expenditures were combined and then adjusted through a pricing index, reconciling 2024 values with those from 2019 to match the emission factor's baseline year. It has been assumed that spend-based emissions factors from the Exiobase database are representative of the organisations, geographies and activities associated.

The comparative data from previous Scope 3 assessments (2022 and 2023) follows the same methodology applied in 2024 for calculating emissions based on capex spend. However, the emission factors used for 2022 and 2023 were sourced from Quantis Scope 3 Evaluator tool, which was in use at the time but has since been discontinued. Similarly to Category One, the emission factors used to calculate the 2022 and 2023 emissions from capital goods  were not updated, as any difference was deemed minimal and not expected to have a material impact.

#### Inclusion and exclusion

The capex spend data from CAML’s accounting software is accurate, with analysis encompassing 98-100% of expenditures per operation.

This category captures capital goods-related emissions associated with CAML’s two primary subsidiaries and operational sites: Kounrad and Sasa.

The following entities have been excluded from the Scope 3 Category 2 assessment, based on considerations of capital expenditure relevance, operational control, and materiality:

- CAML PLC (Head Office): Capital expenditures from the Head Office were excluded due to their minimal contribution to overall capital investments and limited impact on total emissions in this category.

- CAML Exploration (CAML X): An 80%-owned subsidiary established in 2024 for early-stage base metal exploration in Kazakhstan. Given its limited capital activity to date, CAML X has been excluded due to immateriality.

- Aberdeen Minerals (‘Aberdeen’): An associate in which CAML holds a 28.4% interest. As CAML does not have operational control and Aberdeen 's capital investment activities are not considered material, it has been excluded in accordance with the control boundary approach.

The Quantis Scope 3 Evaluator tool is no longer supported and the WRI Scope 3 evaluator tool has been discontinued, prompting a transition to Exiobase for 2024 year’s emissions disclosure. Exiobase was chosen for its broad geographic coverage and detailed industry data, making it well-suited for spend-based emission factor estimates.

### Category 3 – Fuel and energy related activities

| Category 3 |  |
| --- | --- |
| Description | All emissions related to the production of fuels and energy purchased and consumed by CAML in the reporting year, not already included in Scope 1 or Scope 2. This is the upstream emissions from looking at cradle to gate in terms of transportation, creation and extraction of fuels consumed and transmission & distribution (T&D) losses. This includes diesel, gasoline, propane, explosives and electricity across our operations. |
| Emissions (tCO2e) | 5,976 (2023: 6,382) |
| Calculation method | Average-data method |
| Activity data source | Meter readings and consumption data of all energy and fuel related data from the environmental team. |
| Emissions data source | Exiobase database and the World Input – Output Database GHG evaluator. |

#### Boundaries

This category encompasses emissions from the extraction, production, and transportation of fuels and energy used by facilities under CAML’s operational control. It includes upstream emissions related to fuels procured (e.g., diesel or natural gas) for on-site operations and emissions from the extraction, production, and transportation of fuel (e.g., coal or natural gas) burned for purchased electricity. Notably, emissions from fuel combustion at our facilities are categorised as scope 1 emissions, while emissions from purchased electricity generation are considered scope 2 emissions.

#### Calculation method

The volumes of our energy sources were calculated from various sources:

- Litres of diesel and the gasoline from fuel meters readings from our fuel stations.

- kWh of electricity from meter readings across the sites.

- Tonnes of explosives from purchase invoices.

- Litres of propane from purchase invoices

These metrics were then converted into GJ with an emissions factor sourced from the UK Government’s GHG Conversion factors for Company Reporting.

#### Assumptions

We estimated the T&D losses for the electricity procured and utilised at our sites, applying an emission factor, informed by a 2011 study on grid electricity-specific emissions.

#### Inclusion and exclusion

An important distinction in our calculations to note is the exclusion of Transmission and Distribution (T&D) emissions associated with North Macedonia. This exclusion was made deliberately, as the Sasa mine operates entirely on renewable energy sources. As a result, the T&D emissions, amounting to 9,585 tCO2e, were omitted from the calculation, reflecting the mine's commitment to utilising clean energy.

### Category 4 – Upstream transportation and distribution

| Category 4 |  |
| --- | --- |
| Description | All emissions from the transportation and distribution of products (excluding fuel and energy products) purchased or acquired by CAML in the reporting year. This is where CAML pays for the transport costs to sell to our customers. |
| Emissions (tCO2e) | 1,941 (2023: 1,993) |
| Calculation method | Average-data method |
| Activity data source | The activity data provided by our sales department of the number of trucks dispatched during the year including the total weight per truck and distance travelled per journey. |
| Emissions data source | This value was then factored with the specific vehicle emission data provided by the US Environmental Protection Agency. |

#### Boundaries

In accordance with the Scope 3 Standard, this category accounts for emissions categorised as upstream based on financial transactions. The emissions cover road, rail, and marine freight.

#### Calculation method

CAML’s sales team provided details of the total sales to each customer made across the Group. Category 4 includes those transportation and distribution costs paid for by our suppliers. Copper, zinc and lead are transported from the operations to the smelter/customer directly via trucks and a departure port. Distances for each journey were estimated based on reasonable assumptions. For each mode of transport, distances were estimated and emissions factors from the US Environmental Protection Agency for the respective transport mode were applied to the freight distance for an emission estimate of each product.

#### Assumptions

The average-data method used Google Maps to estimate the distance from the operations gate to customers. The data captures the net product mass transported monthly, and discerning the truck types, the latter of which was sourced from our Sales Departments. This data collection process produced the total distance travelled in kilometres and the weight in wet metric tonnes (wmt) per truck.

The average-data method involved multiplying the mass per journey in wmt by the total distance. This value was then factored with the specific vehicle emission data provided by the US Environmental Protection Agency. These figures detailed emissions as CO2, CH4, and N2O per mile. These were then combined  giving an initial value in kilogramme CO2e which was converted into tonnes CO2e.

#### Inclusion and exclusion

No exclusions were made from this category - all sales have been recorded and accounted for. However, the exact distance travelled by each truck and ship has been averaged rather than using  the actual total distance per journey.

### Category 5 – Waste generated in operations

| Category 5 |  |
| --- | --- |
| Description | Emissions arising from third-party external disposal and treatment of waste generated during the reporting year in facilities not controlled by CAML. |
| Emissions (tCO2e) | 480 (2023: 390) |
| Calculation method | Average-data method |
| Activity data source | Waste data provided by the operations Environmental Departments outlining the waste, each sites waste management practices and how waste is categorised into hazardous and non-hazardous waste. |
| Emissions data source | Emission factors for landfills and incinerators taken from the UK Government’s GHG Conversion factors for Company Reporting. Recycling emission factors provided by Turner D. A. et al (2015) study. |

#### Boundaries

Emissions associated with third-party treatment primarily pertain to waste materials transported and disposed off-site. Waste data falls into two categories: hazardous and non-hazardous and is further subcategorised by its disposal method (disposed of in landfill, incinerated or sent for recycling/ reuse).

#### Calculation method

An average-data method, as outlined by the Scope 3 Guidance, was employed. This method utilises emission factors tailored to distinct waste types and their corresponding treatment/disposal methods multiplied by tonnage of material.

Each waste type is assigned an emission factor aligned with its treatment/disposal method. In instances where emission factors for certain treatments were unavailable, the landfill factor was applied, with results presented in kilogramme CO2e per tonne of material.

Emission factors for landfills and incinerators draw from the UK government conversion factors1, a tool adopted by both UK-based and international entities for GHG emissions reporting. Conversely, recycling emission factors derive from studies on GHG emission factors for the recycling of source-segregated waste materials producing a similar output in kilogramme CO2e per tonne of material. It is essential to acknowledge that this reference also offers net emission values in terms of emissions mitigated by reuse, yielding a negative emission output.

#### Assumptions

It was assumed that all materials and respective quantities were hauled off-site and treated in facilities not owned or controlled by the company. Bulk mineral waste excluded, on the basis that it is disposed of on-site.

Distances to the various waste treatment sites were combined for each operation and multiplied by a goods freighting emission factor, therefore the emissions are based on aggregated transportation estimates.

It was assumed that all waste was transported using a heavy goods vehicle.

#### Inclusion and exclusion

Waste analysis includes common recyclables such as mixed paper, mixed plastic, metals, wood, and tires, as well as unique off-site preparations for reuse. Additional items such as oil-based lubricants, light bulbs, car batteries, keck, geomembrane, and office furniture were also included.

### Category 6 – Business travel

| Category 6 |  |
| --- | --- |
| Description | Emissions from the transportation of employees for business-related activities during the reporting year in vehicles not owned or operated by CAML. |
| Emissions (tCO2e) | 292 (2023: 281) |
| Calculation method | Flight and taxi journeys - Distance-based approach Hotel accommodation – Location-based approach |
| Activity data source | The activity data provided by our corporate travel service provider includes details of flights taken, distances travelled and car rental days. |
| Emissions data source | Emissions calculated using industry average emissions factors applied based on flight distance: short, medium, or long-haul. |

#### Boundaries

This category covers emissions from all domestic and international flights undertaken by employees for business travel purposes, as well as other purchased business travel services (taxis, hotels and car rentals) identified from annual spend data. Although these emissions are not considered material, they have been calculated for completeness and accuracy.

#### Calculation method

Total business travel emissions for flights, taxi journeys and hotel accommodation were derived from our travel booking invoices. For flights, a supplier-specific method with estimations derived from airlines, computes emissions per flight based on variables such as, aircraft type, travelled distance, average seating (economy or business class), and passenger load factors. Taxi journey emissions were estimated using a distance-based approach by means of total miles from start to destination. Hotel accommodation emissions were based on total bookings, with an emission factor applied according to average emissions for the country of stay.

#### Assumptions

The airlines’ computations methodology may vary. British Airways, EasyJet and Wizz Air provide an estimated CO2 emissions footprint  per flight. For airlines where no CO2 emissions estimations are provided, the “myclimate” online emission calculator was used to estimate emissions. Ground transportation emissions are based on factors sourced from the UK Department of Transportation, integrating specific vehicle emission factor with travelled distance.

#### Inclusion and exclusion

Assumptions were made for average distances for journeys and country averages for hotel emissions.

### Category 7 – Employee commuting

| Category 7 |  |
| --- | --- |
| Description | Emissions from the transportation of employees between their homes and their worksites during the reporting year in vehicles not owned or operated by CAML. |
| Emissions (tCO2e) | 265 (2023: 240) |
| Calculation method | Average-data approach |
| Activity data source | The data provided by our suppliers of transportation includes details on number of coaches and vans transporting employees to work and the distances travelled. Surveys were carried out for the public transport users to collect data on transport methods and travelled distances. The number of employees commuting by car and the total number of cars parked at our sites were calculated and used. |
| Emissions data source | Transport for London (TFL) emissions disclosure for corporate employees. Operations used total emissions from distance travelled and method of transport. |

#### Boundaries

This category encompasses emissions from the transportation of employees between their homes and their worksites during the reporting year 2024, using vehicles not owned or operated by CAML. While these emissions may not be considered material, they have been estimated for completeness and accuracy in reporting.

#### Calculation method

An average-data approach was employed, estimating the total number of coaches and vans transporting employees to site per month. Additionally, the number of cars parked during shifts over a week was extrapolated over the year to estimate the number of trips taken annually.

During the analysis phase, an average-data approach was adopted. The activity data (distance in kilometre and number of employees) was multiplied by the respective emission factor for each mode of transportation, reported in kilogramme CO2e per kilometre with final figures summed as tCO2e.

While the emission factors for corporate commuters were sourced from the UK government website, those for site-specific employees were derived from internal figures relating to duty cars and coaches.

#### Assumptions

The total number of car journeys by employees was estimated based on the number of cars parked in the car parks of our sites during one shift and extrapolated for the full year.

#### Inclusion and exclusion

No exclusions from this category were made.

### Category 8 – Upstream leased assets

| Category 8 |  |
| --- | --- |
| Description | Emissions from the operation of assets leased that are leased by CAML in the reporting year and not included in scope 1 and scope 2 reported by lessee. |
| Emissions (tCO2e) | - |
| Calculation method | N/A – not material |
| Activity data source | CAML does not have material upstream leased assets and therefore no emissions generated. |
| Emissions data source | N/A |

#### Boundaries

CAML does not have material upstream leased assets and therefore no emissions generated. It is understood that CAML’s emissions relating to its leased assets are quantified in the Scope 1 and 2 inventories.

#### Calculation method

N/A

#### Assumptions

N/A

#### Inclusion and exclusion

N/A

### Category 9 – Downstream transportation & distribution

| Category 9 |  |
| --- | --- |
| Description | All emissions from transportation and distribution of products sold by the company in the reporting year, between CAML’s operations and end consumers, in vehicles and facilities not owned or controlled by the company. This category only includes emissions related to transportation and distribution that occurs after the company pays to produce and distribute its product. |
| Emissions (tCO2e) | 14,573 (2023: 16,025) |
| Calculation method | Average-data method |
| Activity data source | Data provided by our sales department for the number of trucks dispatched during the year including the total weight per truck and distance travelled per journey. |
| Emissions data source | This value was then factored with the specific vehicle emission data provided by the US Environmental Protection Agency. |

#### Boundaries

This category accounts for emissions categorised as downstream based on financial transactions. The emissions cover road, rail, and marine freight.

#### Calculation method

CAML’s sales team provided details of the total sales to each customer made across the Group. Category 9 includes those transportation and distribution costs which are paid for by CAML. Copper, zinc and lead are transported from the operations to the smelter/customer directly via trucks and a departure port. These distances were estimated for each journey based on reasonable assumptions. For each mode of transport, distances were estimated and emission factors from the US environmental protection agency for the respective transport mode were applied to the freight distance to estimate emissions for each product.

#### Assumptions

The average-data method used Google Maps to estimate the distance from the operations gate to customers. The data captured the net product mass transported monthly, and discerning the truck types, the latter of which was sourced from our Head of Sales Departments. This data collection process produced the total distance travelled in kilometres and the weight in wmt per truck.

During analysis, the average-data method was employed: the mass per journey in wmt was multiplied by the total distance. This value was then factored with the specific vehicle emission data provided by the US Environmental Protection Agency1. These figures detailed emissions as CO2, CH4, and N2O per mile. The concluding step aggregated these to produce an initial figure in kilogramme CO2e and converted to tCO2e for summary summation.

#### Inclusion and exclusion

No exclusions from this category were made - all sales have been recorded and accounted for, however, we have used average distance travelled per truck rather than total actual distance travelled.

### Category 10 – Processing of sold products

| Category 10 |  |
| --- | --- |
| Description | All emissions from processing of sold zinc, lead and silver by third parties, after sale by CAML. These are products that require further processing after the company sells them, but before use by the end consumer. |
| Emissions (tCO2e) | 98,659 (2023: 115,647) |
| Calculation method | Supplier-specific method |
| Activity data source | The activity data provided by our sales department of tonnes of zinc, lead and silver to our customers. |
| Emissions data source | Customer-specific emission received from customers |

#### Boundaries

This category involves estimating downstream emissions resulting from the processing of CAML’s sold products, predominantly in concentrate form. This processing often occurs during the pyrometallurgical phase at smelters.

#### Calculation method

The activity data consists of the assessing the cumulative commodity output from both the Kounrad operation and Sasa mine, reported in tonnes. For analysis, a hybrid method is employed. Customer-specific emission factors for sales to customers for zinc, lead and silver have been received, with these factors indicating emissions in terms of tonne CO2 per tonne of commodity. This then been applied using an average-data method for our other customers.

#### Assumptions

As we produce a final copper product at Kounrad, we have assumed no further emissions arise for the processing our copper.

#### Inclusion and exclusion

Emissions from copper processing were excluded since CAML sells a final copper product, that requires negligible further processing. No other exclusions were made in this category, as all sales were recorded and accounted for; however, where data was unavailable, customer-specific data provided by our clients served as the basis for other sales estimates

### Category 11 – Use of sold products

| Category 11 |  |
| --- | --- |
| Description | Emissions from the end use of copper, zinc, lead and silver in the reporting year. |
| Emissions (tCO2e) | 58,339 (2023: 62,708) |
| Calculation method | Average-data method |
| Activity data source | The activity data provided by our sales department of total tonnes of copper, zinc, lead and silver to our various customers during the year. |
| Emissions data source | The International Lead and Zinc Study Group, literature on the lead, copper and silver industries life cycle. |

#### Boundaries

This category is an estimation of the downstream emissions originating from the consumption of sold products beyond the processing stage. These emissions arise as plants and facilities convert the metals into final usable products. For example, the lead concentrate which has been smelted and refined is now cast into commercial-ready form like ingots. These lead ingots then find their way into the hands of manufacturers, such as, a battery manufacturer that  harness the lead to construct both the positive and negative terminals of a lead-based battery.

Calculation method

An average-data method is used for our total tonnes of commodities sold during the reporting year. The emissions for each commodity are estimated using industry data, assumptions have been applied with an average CO2 emissions applied to each tonne of our commodities sold.

#### Assumptions

Due to the lack of precise information regarding the specific uses of our final products, category 11 is based  on industry averages. The International Lead and Zinc Study Group points out that batteries account for the dominant share of lead use at 80%, while rolled products contribute 6%.

The US Environmental Protection Agency reports that zinc is predominantly utilised for galvanising steel. Consequently, emissions associated with zinc usage have been estimated using an emission factor per unit of product.

Similarly, according to the International Copper Association, copper finds extensive applications in buildings, electrical systems, and transportation. Hence, emissions related to copper usage have been estimated using an emission factor per unit of product.

Despite the minimal silver output from our operations, emissions were calculated based on silver entering the jewellery industry, which accounts for 17% of global silver production.

#### Inclusions and exclusions

We couldn't fully account for all sectoral end uses of each commodity because data on end-use emissions is limited. As a result, the sectoral end-use percentages for zinc and lead totalled 67% and 86%, respectively. All uses of CAML’s sold products were based on industry average data, making the total emissions for Category 11 an estimate.

### Category 12 – End-of-Life treatment of sold products

| Category 12 |  |
| --- | --- |
| Description | Emissions from the waste disposal and treatment of copper, zinc, lead and silver at the end of their life. |
| Emissions (tCO2e) | 25,212 (2022: 27,632) |
| Calculation method | Average-data method |
| Activity data source | The activity data provided by our sales department of total tonnes of copper, zinc, lead and silver sold during the reporting year. |
| Emissions data source | - Copper: 0.997 tonnes CO2e per tonne, as determined by a literature on the carbon footprint of both copper and zinc production,. - Zinc: 1.4 tonnes CO2e per tonne, sourced from the Bureau of International Recycling. - Lead: 0.2 tonnes CO2e per tonne, data for recycling was referenced from the US Environmental Protection Agency. |

#### Boundaries

Category 12 accounts for downstream emissions associated with the end-of-life treatment of sold products. Navigating this category presents challenges due to complexities in data availability. The analysis predominantly focuses on the recycling of end-use products, as assumptions about products still in circulation or those that may end up in landfills are speculative and outside the scope of this assessment.

#### Calculation method

The analytical approach employs the average-data method, aligning the recycling emission factor with the specific recycling rates and then applies this to the commodity output.

#### Assumptions

#### Category 12 is estimated using industry averages, as specific data on the end-of-life treatment of sold products is unavailable. Recycling intensities and average CO₂ emissions per tonne of commodity sold are derived from sector-wide sources.

#### Recycling rates were sourced from literature to determine the final metric for each material. For example, Copper Alliance reports a global end-of-life recycling rate for copper at 40%, International Zinc Association cites 34% for zinc, and United Nations Environment Programme provides an average global recycling rate of 71% for lead derived using industry data, and an average CO2 emissions applied to each tonne of our commodities sold.

For the remainder of commodity product not recycled, it is assumed the product is still in circulation, and emissions for landfill are not accounted for.

#### Inclusion and exclusion

Emissions from landfilling are excluded due to limited reliable data, with the analysis focusing instead on end-of-life recycling, where data is more well-documented.

### Category 13 – Downstream leased assets

| Category 13 |  |
| --- | --- |
| Description | Emissions from the operation of assets owned and leased to other entities in the reporting year, not included in Scope 1 and Scope 2. |
| Emissions (tCO2e) | - |
| Calculation method | N/A – Not material |
| Activity data source | CAML does not have downstream leased assets and therefore no emissions generated. |
| Emissions data source | N/A |

#### Boundaries

The emissions inventory boundary is defined to include all assets by CAML and leased onwards for use by an external entity. CAML has not leased any material assets to other entities. It is understood that CAML’s emissions relating to its leased assets are quantified in the Scope 1 and 2 inventories.

#### Calculation method

N/A

#### Assumptions

N/A

#### Inclusion and exclusion

N/A

### Category 14 – Franchises

| Category 14 |  |
| --- | --- |
| Description | Emissions from operation of franchises in the reporting year not included in Scope 1 and Scope 2. |
| Emissions (tCO2e) | - |
| Calculation method | N/A - Not-material |
| Activity data source | CAML does not have franchised operations and therefore no emissions generated. |
| Emissions data source | N/A |

#### Boundaries

CAML does not operate on a franchise model and thus, has no franchisees for which emissions can be attributed.

#### Calculation method

N/A

#### Assumptions

N/A

#### Inclusion and exclusion

N/A

### Category 15 – Investments

| Category 15 |  |
| --- | --- |
| Description | Emissions associated with investment operations during the reporting year that fall outside of Scope 1 and Scope 2. |
| Emissions (tCO2e) | - |
| Calculation | N/A - Not-material |
| Activity data source | No significant emissions produced from investments. |
| Emissions data source | N/A |

#### Boundaries

There are no material emissions arising from our investments during the year ended 31 December 2024.

#### Calculation method

N/A

#### Assumptions

N/A

#### Inclusion and exclusion

The Group hold some investments in exploration companies, however, given limited scope of work at this stage these are immaterial, and the emissions have not been included.

# Appendix 1 - Glossary of terms

Activity Data - quantitative information detailing the specific actions, processes, or events that generate or involve the emission of pollutants, energy consumption, or other relevant environmental metrics, often used in environmental assessments and reporting.

Capital Goods – durable and long-lasting assets, such as machinery, equipment, and buildings, used in the production of goods and services by businesses.

Cradle to Gate – a life cycle assessment approach that evaluates the environmental impact of a product or process from raw material extraction (cradle) to the factory gate, excluding subsequent stages like product use and disposal.

Downstream Emissions - greenhouse gas emissions produced during the distribution, consumption, and disposal phases of a product's life cycle, after its production and prior to the end of its useful life.

Emissions Factor – a numerical representation of the amount of a specific pollutant released into the atmosphere per unit of activity, such as energy production or industrial processes, providing a standard measure for estimating emissions.

Fleet Mobile Combustion – the burning of fossil fuels, such as gasoline or diesel, by a collection of mobile vehicles, like a fleet of cars, trucks, or other transportation means.

Fugitive Emissions - unintended and uncontrolled releases of gases or vapours from industrial processes, equipment, or infrastructure, often escaping into the atmosphere without passing through a designated emission point.

Greenhouse Gas - A greenhouse gas is any gaseous compound, such as carbon dioxide or methane, that absorbs and emits radiation within the thermal infrared range, contributing to the greenhouse effect and influencing the Earth's climate. Greenhouse gases accounted for in this report are the seven main gases covered by the UNFCCC/Kyoto Protocol as defined in GHG Protocol, as amended: carbon dioxide (CO2); methane (CH4); nitrous oxide (N2O); hydrofluorocarbons (HFCs); perfluorocarbons (PFCs); sulphur hexafluoride (SF6); and nitrogen trifluoride (NF3).

GHG Protocol - Greenhouse Gas Protocol Corporate Value Chain (Scope 3) Accounting and Reporting Standard Supplement to the GHG Protocol Corporate Accounting and Reporting Standard, Copyright of World Resources Institute and World Business Council for Sustainable AA 2021 Scope 3 Methodology Summary Report Development.

Global Reporting Initiative - a non-profit organisation that promotes sustainability reporting standards and provides a framework for organisations to transparently communicate their economic, environmental, and social performance to foster global sustainability.

Location-based Scope 2 Emissions - indirect GHG emissions associated with the generation of purchased or acquired electricity, steam, heat, or cooling consumed by an organisation's operations at a specific location. These emissions occur off-site at the location where the energy is generated but are attributed to the organisation based on its consumption.

Scope 1 Emissions - encompasses direct greenhouse gas emissions originating from CAML’s own activities or sources under its control. Managing these emissions is essential for organisations committed to reducing their environmental footprint and promoting sustainability.

Scope 2 Emissions - Indirect greenhouse gas emissions stemming from purchased energy sources, such as electricity and heat. Understanding and effectively managing Scope 2 emissions is crucial for organisations seeking to minimise their environmental impact and enhance overall sustainability efforts.

Scope 3 Emissions - Indirect GHG emissions that occur in the value chain of a company but are not directly owned or controlled by that company. These encompass all indirect emissions associated with its operations in its upstream and downstream activities.

Stationary Combustion - the process of burning fuels, typically in industrial facilities or power plants, where the combustion source remains fixed in one location rather than being mobile.

Upstream Emissions - greenhouse gas emissions associated with the extraction, production, and transportation of raw materials or fuels before they reach the point of use or consumption in a specific process or product.

# Appendix 2 - Units of measurement

GJ - Gigajoule

kWh – Kilowatt per hour

kg - Kilogrammes

CO2e – Carbon dioxide equivalent

tCO2e - Metric tonnes of carbon dioxide equivalent

wmt – Wet metric tonnes

km - Kilometres

t – Tonnes

# Appendix 3 – The Scope 1 and 2 emission sources

## Table 3.1 – The Sources of CAML’s Scope 1 emissions

|  | Kounrad |  |  | Sasa |  |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 |
|  | GJ | GJ | GJ | GJ | GJ | GJ | GJ | GJ | GJ |
| Diesel | 6,958 | 8,913 | 8,050 | 48,230 | 41,935 | 39,354 | 55,188 | 50,848 | 47,404 |
| Gasoline | 4,838 | 5,403 | 5,453 | - | - | - | 4,838 | 5,403 | 5,453 |
| Propane | - | 5 | 1 | - | - | - | - | 5 | 1 |
| Explosives | - | - | - | 3,041 | 2,970 | 2,933 | 3,041 | 2,970 | 2,933 |
| Coal | 267,116 | 247,160 | 247,154 | - | - | - | 267,116 | 247,160 | 247,154 |
| Total | 278,912 | 261,481 | 260,658 | 51,271 | 44,905 | 42,287 | 330,183 | 306,386 | 302,945 |

## Table 3.2 – Sources of Scope 2 emissions

|  | Kounrad |  |  | Sasa |  |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 | 2024 | 2023 | 2022 |
|  | GJ | GJ | GJ | GJ | GJ | GJ | GJ | GJ | GJ |
| Purchased electricity | 182,264 | 209,820 | 217,831 | 166,794 | 159,936 | 155,669 | 349,058 | 369,756 | 373,500 |
| Total | 211,318 | 212,373 | 217,831 | 166,794 | 159,936 | 155,669 | 349,058 | 369,756 | 373,500 |

# Appendix 4 – References

- Brander, Matthew, et al (2011). ‘Electricity-Specific Emission Factors for Grid Electricity’,
https://ecometrica.com/assets/Electricity-specific-emission-factors-for-grid-electricity.pdf

- British Airways (2024), Estimate Your Carbon Emissions  https://ba.chooose.today/

- Bureau of International Recycling (2008), ‘Report on the Environmental Benefits of Recycling’.
https://www.mgg-recycling.com/wp-content/uploads/2013/06/BIR_CO2_report.pdf

- Carbon Disclosure Project (2023), ‘CDP Technical Note: Relevance of Scope 3 Categories by Sector’ https://cdn.cdp.net/cdp-production/cms/guidance_docs/pdfs/000/003/504/original/CDP-technical-note-scope-3-relevance-by-sector.pdf

- Carbon Footprints of Teck Special High Grade Zinc and Continuous Galvanizing Grade Zinc (2023) Carbon-Footprint-of-Teck-Special-High-Grade-Zinc.pdf

- Davidson, Alistair, et al (2016). ‘Environmental Impact and LCA Of Lead Battery and Architectural Sheet Production’,
https://link.springer.com/article/10.1007/s11367-015-1021-5

- Department for Energy Security and Net Zero (2024), ‘2024 Government Greenhouse Gas (GHG) Conversion Factors for Company Reporting’ , UK Govt. 
https://www.gov.uk/government/publications/greenhouse-gas-reporting-conversion-factors-2024

- Department for Transportation (2021), ‘Vehicle licensing statistics data tables’, UK Govt. https://committees.parliament.uk/publications/7590/documents/79606/default/

- Easy Jet (2024), Calculate Footprint https://easyjet.southpole.com/

- Greenhouse Gas Emission in Jewelry Industry: A Case Study of Silver Flat Ring (2016) https://www.thaiscience.info/Journals/Article/APER/10979994.pdf?form=MG0AV3

- Greenhouse Gas Protocol (2011), ‘Corporate Value Chain Accounting and Reporting Standard’, World Resources Institute.
https://ghgprotocol.org/sites/default/files/standards/Corporate-Value-Chain-Accounting-Reporing-Standard_041613_2.pdf

- Greenhouse Gas Protocol (2011), ‘Technical Guidance for Calculating Scope 3 Emissions’, World Resources Institute.
https://ghgprotocol.org/sites/default/files/standards/Scope3_Calculation_Guidance_0.pdf

- International Copper Association (2023), ‘Copper – The Pathway To Net Zero’, Copper Alliance.
https://copperalliance.org/wp-content/uploads/2023/02/ICA-GlobalDecarbonization-202301-Final-singlepgs.pdf

- International Standard Industrial Classification of All Economic Activities (ISIC), Revision 3.1 https://unstats.un.org/unsd/classifications/Econ/Download/In%20Text/Isic31_English.pdf

- My Climate (2024), Flight CO2 Tool.
https://co2.myclimate.org/en/flight_calculators/new

- Nilsson, Anna, et al (2017). ‘A Review of the Carbon Footprint of Cu and Zn Production from Primary and Secondary Sources,
https://www.researchgate.net/publication/319697995_A_Review_of_the_Carbon_Footprint_of_Cu_and_Zn_Production_from_Primary_and_Secondary_Sources

- Quantis (2023), Quantis Scope 3 Evaluator Tool.
https://quantis-suite.com/Scope-3-Evaluator/

- Stadler, K. W.-J.-F. (2021), EXIOBASE 3 (3.8.2).
https://doi.org/10.5281/zenodo.5589597

- Turkish Airlines (2024), CO2mission (carbon footprint calculator) https://turkishairlines.co2mission.com/en/offset/calculation

- Turner A, D. et al (2015). ‘Greenhouse gas emission factors for recycling of source-segregated waste materials’,
https://www.sciencedirect.com/science/article/pii/S0921344915301245?via%3Dihub

- Transport For London (2023), ‘TfL CO2 emissions per passenger kilometre’, tfl.gov.uk.
https://foi.tfl.gov.uk/FOI-3373-2324/FOI-3373.pdf

- U.S. Bureau of Economic Analysis (2025), ‘Chain-Type Quantity Indexes for Gross Output by Industry’, bea.gov.
https://apps.bea.gov/iTable/?reqid=150&step=2&isuri=1&categories=gdpxind#eyJhcHBpZCI6MTUwLCJzdGVwcyI6WzEsMiwzXSwiZGF0YSI6W1siY2F0ZWdvcmllcyIsIkdkcHhJbmQiXSxbIlRhYmxlX0xpc3QiLCIxOCJdXX0=

- U.S. Environmental Protection Agency (2023), ‘Emissions From Hit-Dip Galvanizing Process’, NSCEP. https://nepis.epa.gov/Exe/tiff2png.cgi/94001BY2.

- U.S. Environmental Protection Agency (2025), ‘GHG Emission Factors Hub’, epa.gov. https://www.epa.gov/system/files/documents/2025-01/ghg-emission-factors-hub-2025.pdf

- U.S. Environmental Protection Agency (1997), ‘Secondary Lead Processing’, epa.gov. https://www3.epa.gov/ttnchie1/ap42/ch12/final/c12s11_2010.pdf

- U.S. Environmental Protection Agency (2023), ‘Used Lead Acid Batteries – Waste Lead Acid Batteries’, epa.gov.
https://www.unep.org/explore-topics/chemicals-waste/what-we-do/emerging-issues/used-lead-acid-batteries-ulab-waste-lead.

- U.S. Geological Survey (2020), ‘Mineral Commodity Summaries’, usgs.gov.
https://pubs.usgs.gov/periodicals/mcs2020/mcs2020.pdf

- Wizz Air (2024), Carbon Footprint Calculator. https://wizzair.chooose.today/

The emissions sources for the CAML’s Scope 1 can be found in Appendix 3.

The emissions sources for the CAML’s Scope 2 can be found in Appendix 3.

Greenhouse Gas Protocol (2011) https://ghgprotocol.org/sites/default/files/standards/Corporate-Value-Chain-Accounting-Reporing-Standard_041613_2.pdf

Greenhouse Gas Protocol (2013) https://ghgprotocol.org/sites/default/files/2022-12/Required%20gases%20and%20GWP%20values_0.pdf

Greenhouse Gas Protocol (2013) https://ghgprotocol.org/sites/default/files/2023-03/Scope3_Calculation_Guidance_0%5B1%5D.pdf

U.S. Bureau of Economic Analysis (2024) https://apps.bea.gov/iTable/?reqid=150&step=2&isuri=1&categories=gdpxind#eyJhcHBpZCI6MTUwLCJzdGVwcyI6WzEsMiwzXSwiZGF0YSI6W1siY2F0ZWdvcmllcyIsIkdkcHhJbmQiXSxbIlRhYmxlX0xpc3QiLCIxOCJdXX0=

Stadler, K. W.-J.-F. (2021) https://doi.org/10.5281/zenodo.5589597

U.S Bureau of Economic Analysis (2024) https://apps.bea.gov/iTable/?reqid=150&step=2&isuri=1&categories=gdpxind#eyJhcHBpZCI6MTUwLCJzdGVwcyI6WzEsMiwzXSwiZGF0YSI6W1siY2F0ZWdvcmllcyIsIkdkcHhJbmQiXSxbIlRhYmxlX0xpc3QiLCIxOCJdXX0=

U.S Bureau of Economic Analysis (2024) https://apps.bea.gov/iTable/?reqid=150&step=2&isuri=1&categories=gdpxind#eyJhcHBpZCI6MTUwLCJzdGVwcyI6WzEsMiwzXSwiZGF0YSI6W1siY2F0ZWdvcmllcyIsIkdkcHhJbmQiXSxbIlRhYmxlX0xpc3QiLCIxOCJdXX0=

Stadler, K. W.-J.-F. (2021) https://doi.org/10.5281/zenodo.5589597

Quantis (2023) https://quantis-suite.com/Scope-3-Evaluator/

Quantis (2023) https://quantis-suite.com/Scope-3-Evaluator/

Stadler, K. W.-J.-F. (2021) https://doi.org/10.5281/zenodo.5589597

Stadler, K. W.-J.-F. (2021) https://doi.org/10.5281/zenodo.5589597

U.S Bureau of Economic Analysis (2024) https://apps.bea.gov/iTable/?reqid=150&step=2&isuri=1&categories=gdpxind#eyJhcHBpZCI6MTUwLCJzdGVwcyI6WzEsMiwzXSwiZGF0YSI6W1siY2F0ZWdvcmllcyIsIkdkcHhJbmQiXSxbIlRhYmxlX0xpc3QiLCIxOCJdXX0=

Stadler, K. W.-J.-F. (2021) https://doi.org/10.5281/zenodo.5589597

Quantis (2023) https://quantis-suite.com/Scope-3-Evaluator/

Stadler, K. W.-J.-F. (2021) https://doi.org/10.5281/zenodo.5589597

U.S Bureau of Economic Analysis (2024) https://apps.bea.gov/iTable/?reqid=150&step=2&isuri=1&categories=gdpxind#eyJhcHBpZCI6MTUwLCJzdGVwcyI6WzEsMiwzXSwiZGF0YSI6W1siY2F0ZWdvcmllcyIsIkdkcHhJbmQiXSxbIlRhYmxlX0xpc3QiLCIxOCJdXX0=

Brander, Matthew, et al (2011). https://ecometrica.com/assets/Electricity-specific-emission-factors-for-grid-electricity.pdf

U.S. Environmental Protection Agency (2025)https://www.epa.gov/system/files/documents/2025-01/ghg-emission-factors-hub-2025.pdf

U.S. Environmental Protection Agency (2025) https://www.epa.gov/system/files/documents/2025-01/ghg-emission-factors-hub-2025.pdf

Department for Energy Security and Net Zero (2024)
https://www.gov.uk/government/publications/greenhouse-gas-reporting-conversion-factors-2024

Turner D. A. et al (2015) https://www.sciencedirect.com/science/article/pii/S0921344915301245?via%3Dihub

Greenhouse Gas Protocol (2013) https://ghgprotocol.org/sites/default/files/2023-03/Scope3_Calculation_Guidance_0%5B1%5D.pdf

British Airways (2024) https://ba.chooose.today/

EasyJet via South Pole (2024) https://easyjet.southpole.com/

Wizz Air (2024) https://wizzair.chooose.today/

myclimate (2024) https://co2.myclimate.org/en/flight_calculators/new

Department for Transportation (2021) https://committees.parliament.uk/publications/7590/documents/79606/default/

Transport For London (2023) https://foi.tfl.gov.uk/FOI-3373-2324/FOI-3373.pdf

U.S. Environmental Protection Agency (2025) https://www.epa.gov/system/files/documents/2025-01/ghg-emission-factors-hub-2025.pdf

Davidson, Alistair, et al (2016) https://link.springer.com/article/10.1007/s11367-015-1021-5

U.S. Environmental Protection Agency (2023) https://www.unep.org/explore-topics/chemicals-waste/what-we-do/emerging-issues/used-lead-acid-batteries-ulab-waste-lead

International Copper Association (2023) https://copperalliance.org/wp-content/uploads/2023/02/ICA-GlobalDecarbonization-202301-Final-singlepgs.pdf

Greenhouse Gas Emission in Jewelry Industry: A Case Study of Silver Flat Ring (2016)
https://www.thaiscience.info/Journals/Article/APER/10979994.pdf?form=MG0AV3

Davidson, Alistair, et al (2016)

U.S. Environmental Protection Agency (2023) https://nepis.epa.gov/Exe/tiff2png.cgi/94001BY2

International Copper Association (2023) https://copperalliance.org/wp-content/uploads/2023/02/ICA-GlobalDecarbonization-202301-Final-singlepgs.pdf

Greenhouse Gas Emission in Jewelry Industry: A Case Study of Silver Flat Ring (2016)
https://www.thaiscience.info/Journals/Article/APER/10979994.pdf?form=MG0AV3

Bureau of International Recycling (2008) https://www.mgg-recycling.com/wp-content/uploads/2013/06/BIR_CO2_report.pdf

Nilsson, Anna, et al (2017) https://www.researchgate.net/publication/319697995_A_Review_of_the_Carbon_Footprint_of_Cu_and_Zn_Production_from_Primary_and_Secondary_Sources

Bureau of International Recycling (2008)

U.S. Environmental Protection Agency (1997) https://www3.epa.gov/ttnchie1/ap42/ch12/final/c12s11_2010.pdf

International Copper Association (2023) https://copperalliance.org/wp-content/uploads/2023/02/ICA-GlobalDecarbonization-202301-Final-singlepgs.pdf

Bureau of International Recycling (2008) https://www.mgg-recycling.com/wp-content/uploads/2013/06/BIR_CO2_report.pdf

U.S. Environmental Protection Agency (1997) https://www3.epa.gov/ttnchie1/ap42/ch12/final/c12s11_2010.pdf

Please note, that the values show the purchased electricity from the grid.